PROPERTY DESCRIPTION

Being all that tract or parcel of land lying and being in Land Lot 361, 18th District, DeKalb County, Georgia and being more particularly described as follows:

Beginning for the same at a ½ inch rebar found on the Easterly Right-of-Way Line of Vermack Road (an 80 feet wide right of way), said point being at State Plane Coordinate (Georgia West Zone) of North: 1,432,664.332; East: 2,251,573.065; thence, leaving said Point of Beginning and running with the said line of Vermack Road

- 70.06 feet along the arc of a curve deflecting to the left, having a radius of 1,343.55 feet and a chord bearing and distance of North 02° 14' 24" West, 70.05 feet; thence,

- North 03° 41' 48" West, 194.92 feet to a ½ inch rebar found; thence, leaving the aforesaid line of Vermack Road and running adjacent to the property now or formerly owned by Paul Y. Shane and Sandra Shane, as described in a deed recorded among the Land Records of DeKalb County, Georgia in Deed Book 7165, Page 327, and the lines of a subdivision entitled, “Village Mill, Unit Two”, as shown on a plat recorded among the aforesaid Land Records in Plat Book 54, Page 69,

- North 89° 32' 56" East, 569.60 feet to a ½ inch rebar found; thence,

- South 00° 05' 20" East, 243.12 feet to a ½ inch rebar found; thence, leaving the aforesaid lines of the Village Mill subdivision and running,

- South 89° 16' 05" West, 234.50 feet to a ½ inch rebar found; thence,

- South 75° 18' 21" West, 151.29 feet to a ½ inch rebar found; thence,

- North 84° 54' 19" West, 174.52 feet to the Point of Beginning, containing 144,865 square feet or 3.3256 acres of land, more or less.

Property is subject to all easements and rights of way recorded and unrecorded.